Gram Staining SOP

Supplies

- Specimen culture to be smeared

- Inoculation loops

- Glass slide

- Permanent marker

- Deionized water

- Tupperware container

- Glass slide holder

- Heat fixation device

- Gram Stain Kit

  - Crystal Violet Reagent

  - Iodine Reagent

  - Decolorizer

  - Advanced Counterstain

- Kimwipes

- Microscope

- Immersion oil

Methods

- Draw a circle with a permanent marker on the glass slide to indicate where specimen has been smeared, and attach metal slide holder to slide.

- Apply a drop (about 10 uL) of deionized water to the area within the circle previously drawn. Tilt glass slide as necessary to ensure water covers the entire area of the circle.

- Collect the test specimen from culture with inoculation loop.

- Apply the test specimen to the glass slide in the water within the circle.

- With a gas burner, heat fix the smear to the slide by passing the slide through a low flame several times until all moisture has evaporated. Allow slide to cool to room temperature before staining.

- Over the Tupperware container, cover the slide with the Crystal Violent Reagent for one minute.

- Rinse the slide with deionized water, while avoiding to rinse directly inside the circle with the specimen.

- Cover the slide with the Iodine Reagent for one minute.

- Gently rise the slide with deionized water and allow to drain.

- Tilt the slide and flood the slide with a few drops of Decolorizer until no violet color runs off. This will usually take 10 seconds or less. Do not over decolorize.

- Rinse slide gently with deionized water.

- Cover slide with Advanced Counterstain for one minute.

- Rinse slide gently with deionized water. The rinse water in this step should be slightly pink. Do not over wash.

- Allow slide to drain and air dry, or gently dry with Kimwipes.

- Examine the slide on the microscope, first under 40X magnification. Adjust as necessary.

- Increase magnification to 100, and apply drop of immersion oil to lens.

- Slowly adjust microscope so 100X magnification lens gently touches the glass slide to avoid cracking/breaking the slide.

- Examine specimen and adjust as necessary.