MAC-112, Machining Technology II

Quiz Questions and Answers:

Week 1:

What should employers prioritize to protect employees from workplace hazards?

Providing comprehensive healthcare plans

Using all feasible engineering and work practice controls*

Offering bonuses for completing safety training

Encouraging employees to bring their own PPE

points:1.00

When must employers provide PPE to employees?

Only if the employees request it

At the employees' expense

At no cost to employees, except for specific items*

Only for high-risk jobs

points:1.00

What is an example of an engineering control to eliminate workplace hazards?

Providing safety training to employees

Using personal hygiene practices

Substituting less harmful materials*

Encouraging job rotation of workers

points:1.00

What must employees be trained on regarding PPE?

How to design new PPE

Proper care, maintenance, and disposal of PPE*

How to request free PPE from the employer

The brand names of different PPE items

points:1.00

Which of the following is a cause of foot injuries that safety shoes can protect against?

Slippery surfaces*

Loud noises in the workplace

Bright lights

Extreme temperatures

points:1.00

What is the proper disposal method for shop rags that come in contact with general shop liquids?

Throw them in any garbage bin

Leave them on the floor for the janitor to pick up

Store them in your locker for reuse later

Dispose of them in the red metal garbage cans with foot pedal opening mechanism*

points:1.00

Who are considered Authorized Employees in Lockout/Tagout procedures?

Machine operators who use the equipment daily

Maintenance staff who perform the work and apply the lockout/tagout devices*

Employees who come into the area where the work is being done

All employees in the facility

points:1.00

What are the four main types of electrical injuries?

Cuts, bruises, burns, falls

Electrocution, electrical shock, burns, falls*

Trips, slips, fractures, concussion

Chemical burns, radiation exposure, heat exhaustion, lacerations

points:1.00

What is the purpose of a Ground-Fault Circuit Interrupter (GFCI)?

To detect high voltage in electrical circuits

To prevent overloads in electrical systems

To protect against dangerous electrical shocks*

To regulate the flow of current in appliances

points:1.00

Which of the following is NOT a clue that electrical hazards exist?

Tripped circuit breakers or blown fuses

Warm tools, wires, cords, connections, or junction boxes

GFCI that shuts off a circuit

Employees wearing proper protective equipment*

points:1.00

What should be done before inspecting or making repairs on electric equipment?

Turn on all power sources to ensure proper function

Use flexible cords instead of fixed wiring

Deenergize the electric equipment*

Increase the voltage for better performance

points:1.00

What is one potential danger of using flexible cords improperly?

They are more durable than fixed wiring

They are less susceptible to damage

They can cause shocks, burns, or fires*

They are not approved by OSHA regulations

points:1.00

What are the four main types of electrical injuries mentioned in the PowerPoint?

Electrocution, electrical shock, burns, falls*

Dizziness, disorientation, fainting, nausea

Sprains, bruises, lacerations, concussions

Strains, fractures, burns, cuts

points:1.00

What is the purpose of grounding in electrical systems?

To increase voltage

To reduce current flow

To provide a conductive connection to the earth for protection*

To decrease resistance

points:1.00

What is the primary danger of inadequate wiring in electrical systems?

Risk of fire due to overheating*

Increased voltage levels

Higher resistance in the circuit

Reduced energy consumption

points:1.00

Which type of machine guard is described as a movable barrier device that protects the operator at the point of operation before the machine cycle can be started?

Fixed guard

Interlocked guard

Adjustable guard

Gate*

points:1.00

What are some responsibilities of employees regarding machine safety, according to the text

Ensure all machinery is properly guarded

Train employees on specific guard rules

Immediately correct machine guard deficiencies

Report machine guard problems to supervisors immediately*

points:1.00

Week 2:

What is the primary function of an angle grinder?

Shaping metal parts

Polishing cylindrical surfaces

Cutting, deburring, and cleaning*

Sharpening cutting tools

points:1.00

What is the typical power rating range for bench grinders?

100-500 W

500-3000 W*

3000-5000 W

5000-10,000 W

points:1.00

Which type of grinder is commonly used for fine detailing and light work?

Die Grinder*

Belt Grinder

Bench Grinder

Centerless Grinder

points:1.00

What is a characteristic feature of hand grinders?

They are fixed to workstations.

They are used for heavy-duty grinding.

They accept different attachments like grinding discs and wire wheels.*

They are typically operated with both hands

points:1.00

Which type of grinder is specifically made for sharpening cutting tools such as drill bits and milling cutters?

Belt Grinder

Precision Grinder

Jig Grinder

Tool and Cutter Grinder*

points:1.00

What are the three different interactions that occur between the abrasive and the machined material during grinding?

Cutting, welding, sliding

Cutting, plowing, sliding*

Cutting, polishing, sliding

Cutting, chipping, sliding

points:1.00

Which type of grinding maximizes metal removal but sacrifices surface finish?

Precision grinding

Ultra-precision grinding

Rough grinding*

Creep feed grinding

points:1.00

What is the primary purpose of creep feed grinding?

To make deep cuts in steel materials at low speeds*

To achieve ultra-precision surface finish

To maximize metal removal at high speeds

To remove unwanted metal with little concern for surface finish

points:1.00

What is the purpose of superfinishing in the grinding process?

To remove material quickly

To improve surface finish and remove peaks and valleys*

To maximize metal removal

To achieve ultra-precision cuts

points:1.00

Week 3:

True or False: The metal removal rate for a milling machine is higher than that of a lathe.

True*

False

points:1.00

Which of the following parts of a milling machine is the main supporting frame mounted on the base?

Knee

Column*

Spindle

Saddle

points:1.00

Which of the following parts of a milling machine sits on top of the knee?

Base

Column

Arbor

Saddle*

points:1.00

True or False: the accuracy of the cutter depends of the strength, accuracy, and rigidity of the arbor.

True

False*

points:1.00

True or False: for a milling machine, the work can only be fed in a longitudinal direction?

True

False*

points:1.00

Which of the following is NOT a type of milling machine?

Hand milling machine

Horizontal milling machine

Diagonal milling machine*

Vertical milling machine

points:1.00

True or False: the lathe machine rotates the workpiece about an axis to perform different operations.

True*

False

points:1.00

From the options below, select the types of operations performed on a lathe (you can select multiple answers if needed)

Turning*

Facing*

Chamfering*

Drilling*

points:1.00

Which of the following is not a main part of a lathe machine?

Bed

Headstock

Feed Rod

Carriage

Footstock*

points:1.00

Which of the following are types of lathe machines?

Speed lathe machine*

Spindle lathe machine

Bench lathe machine*

Carriage lathe machine

points:1.00

Week 4:

What type of chuck is pictured above?

points:1.00

What type of chuck is shown in the picture above?

points:1.00

What mounting type is shown in the picture above?

points:1.00

What type of mount is shown in the picture above?

points:1.00

What type of mount is shown in the picture above?

points:1.00

Week 5:

Week 5’s Quiz is linked in the main course PDF.

Week 6:

Week 6’s Quiz/Worksheet is linked in the main course PDF.