MAC 111 Exam Questions and Answers

Mid-Term Exam Questions:

What are some common hazards associated with band saws?

Electric shock and burns

Falling objects and slippery surfaces

Contact with blade, nip points, kickback*

Exposure to intense light and chemicals

points:1.00

Which of the following is NOT a recommended safety control for drill presses?

Using appropriate PPE

Removing gloves before operating the drill press

Adjusting the drill press while the drill bit is rotating*

Securing material to the drill press bed with clamps

points:1.00

What is one primary safeguarding method recommended for drill press operations?

Using barricades to separate employees

Wearing gloves and loose clothing

Installing guarding over motor, belts, and pulleys*

Adjusting the drill press while it is in operation

points:1.00

What is the purpose of spark-resistant tools in the workplace?

To prevent slips and trips

To reduce the risk of static electricity buildup*

To protect against falling objects

To avoid distractions while working

points:1.00

Which safety guideline is recommended for using cutting tools?

Carry sharp tools in your pocket

Test tool sharpness with fingers

Use dull blades for cutting

Start the cut close to your body, then cut outward*

points:1.00

What should you do before using any hand tool?

Inspect it for wear and damage*

Use it without checking for safety

Modify the tool if needed

Use the tool for multiple operations at once

points:1.00

What is the primary function of a power saw?

Grinding material into smaller pieces

Cutting material to a desired length or contour*

Shaping material using heat and pressure

Forming material through compression

points:1.00

Which safety precaution should be observed when using a vertical band saw?

Use gloves to handle band saw blades

Keep fingers well clear of the blade at all times*

Adjust the band saw blade while it is in operation

Use the band saw without a miter guide attachment

points:1.00

What is the purpose of abrasive cutoff saws?

Producing curled chips for metal cutting

Cutting material too hard for a metal saw*

Making accurate square or mitered cuts

Providing a continuous band blade for cutting

points:1.00

What should you do immediately after using the chuck key or drill drift on a drill press?

Leave it in the chuck or spindle

Store it on the drill press table

Remove it from the chuck or spindle*

Use it to remove hot tools

points:1.00

True or False: the above drill bit picture is a countersink.

True

False*

points:1.00

Which of the following is not a type of drill press drive?

Cone pully drive

Spiral pulley drive*

Variable speed drive

Gear head drive

points:1.00

Which of the following are operations performed on a drill press? (select all that apply)

Drilling*

Boring*

Tapping*

Sawing

points:1.00

True or False: All workholding devices must be secured to the table with clamps or bolts.

True*

False

points:1.00

What does Cutting Speed (CS) refer to in machining?

The rate at which the tool is moved into the part

The rate at which the part is moved into the tool

The circumferential speed of the milling cutter*

The number of teeth on the cutter

points:1.00

How is Feed rate measured in machining? Select all that apply.

Feet per minute (FPM)

Inches per revolution (IPR)*

Revolutions per minute (RPM)

Inches Per Minute (IPM)*

points:1.00

What formula is used to convert Cutting Speed (CS) to Spindle Speed (RPM)?

RPM = (CS × Diameter of cutter) / 3.82

RPM = (4 × CS) / (Diameter of cutter)*

RPM = (CS × Diameter of cutter) / 2.54

RPM = (CS × Diameter of cutter) / 1.62

points:1.00

What is one of the primary safeguarding methods recommended for protecting operators from milling machine hazard areas?

Using gloves and loose-fitting clothing

Installing guards around power transmission components*

Checking the cutter rotation while the machine is running

Ignoring warning signs and barriers

points:1.00

What should operators avoid doing while the milling machine is in motion or not locked or tagged out?

Cleaning chips or adjusting coolant*

Using proper safeguarding devices

Wearing gloves and jewelry

Engaging the machine way oil pump

points:1.00

What milling machine accessory is pictured above?

Precision vise*

Vee block and clap

Hold-down clamp

Angle plate

points:1.00

True or False: the above image show a double end cutting tool.

True*

False

points:1.00

Match the machining operation with its definition/description

Turning

Removing material from outside surface of part

Facing

Removing material from the end surface of part

Grooving

Cutting a recess in work

Boring

Enlarging an existing hole

Chamfering

Producing a flat, angular surface on the edge of a machined part

Knurling

Producing a raised surface of work to improve grip or appearance or to increase diameter

Treading

Forming internal or external threads with a single-point tool

Drilling

Producing a hole with a drill

Reaming

Enlarging a hole slightly with a multi-fluted cutter

Forming

Producing a desired shape by using tools to ground that shape

Shouldering

Fishing surface where two diameters or steps meet

Tapering

Producing uniform change in diameter of a part as measured along its axis

Filling

Chamfering or reducing diameter of work with a file while work is rotating

Polishing

Improving surface of part with abrasive cloth while work is turning

Cutoff

Cutting a part from work with a thin tool while work is rotating

Tapping

Producing internal threads with a tap, using the lathe tailstock to guide tap

Center Drilling

Producing a tapered hole in end of work so that part can be held between centers or be used as starting hole for drilling operation

points:17.00

Match with lathe machine part with it's corresponding letter above. A [blank], B [blank], C [blank], D [blank]

Headstock

Bed

Tailstock

Carriage

points:1.00

Final Exam:

What primary safeguarding method can be installed to guard the operator and other employees from rotating parts, flying chips, and cuttings on a drill press?

Barricades

Automatic machines

Shields attached to the quill*

Conducting periodic inspections

points:1.00

What is a secondary safeguarding method that may be used to supplement primary safeguarding methods for drill presses?

Removing the chuck after each use*

Adjusting the drill press while the bit is rotating

Using gloves, jewelry, and loose-fitting clothing while operating the drill press

Wearing long hair loose during drilling operations

points:1.00

What type of eye protection must employers provide for employees exposed to potential eye injuries during drill press work?

Welding shields

Safety spectacles*

Conductive shoes

Face shields

points:1.00

What is a recommended guideline for using hand tools safely according to the provided text?

Use tools with dull blades for safer handling

Carry sharp tools in your pocket for easy access

Always alter a tool to fit the specific task at hand

Remove cracked saw blades from service*

points:1.00

What is a key safety guideline when operating a bandsaw?

Wear loose-fitting clothing and jewelry for comfort.

Keep the blade guard open for better visibility.

Use push sticks or devices to keep hands away from the blade.*

Adjust the blade tension while the bandsaw is running.

points:1.00

True or false: face shields can protect you from impact hazards

True

False*

points:1.00

What is the primary function of a power saw?

To drill holes in materials

To shape materials into desired contours*

To fasten materials together

To polish surfaces

points:1.00

What precaution should be taken when handling band saw blades?

Use bare hands for a better grip.

Handle with care as large springy blades can be dangerous.*

Use gloves to protect hands from the blade.

Remove the power supply after handling blades.

points:1.00

What is the difference between a horizontal and vertical band saw?

Horizontal saws use continuous band blades, while vertical saws use toothed blades.

Horizontal saws lower the blade onto the material, while vertical saws feed the material into the blade.*

Horizontal saws have a coolant reservoir, while vertical saws do not.

Horizontal saws are faster than vertical saws.

points:1.00

What is the purpose of using a miter guide attachment or work-holding device with a power saw?

To increase the cutting speed

To reduce the noise level

To keep fingers well clear of the blade*

To cool down the blade during operation

points:1.00

What type of saw is recommended for cutting materials too hard for a metal saw to cut?

Power saw

Cold saw

Band saw

Abrasive cutoff saw*

points:1.00

True or False: Cone pulley drives are found on floor mounted machines like box column and radial drill presses.

True

False*

points:1.00

True or False: the above picture is an image of a tapping attachment.

True*

False

points:1.00

True or False: the above picture is an image of a straight shank twist drill.

True

False*

points:1.00

Which of the following is a workholding/set-up device used in drill press operations?

Angle Vise*

J-Clamp

T-Block

S-Bolt

points:1.00

What is Cutting Speed (CS) in milling expressed as?

Inches per minute (IPM)

Revolutions per minute (RPM)

Surface feet per minute (SFM)*

Feet per tooth (FTP)

points:1.00

How do you convert Cutting Speed (CS) to Spindle Speed (RPM) for milling?

Divide CS by the diameter of the cutter

Multiply CS by 4 and divide by the diameter of the cutter*

Divide the diameter of the cutter by CS

Multiply CS by the number of teeth on the cutter

points:1.00

What is Feed Rate (F) measured in for milling?

Inches per tooth (IPT)

Surface feet per minute (SFM)

Inches per revolution (IPR)

Inches per minute (IPM)*

points:1.00

How do you calculate Feed Rate (F) per tooth for milling?

Multiply Feed Rate (F) by the number of teeth on the cutter

Divide Feed Rate (F) by the number of teeth on the cutter*

Divide Feed Rate (F) by the diameter of the cutter

Multiply Feed Rate (F) by the diameter of the cutter

points:1.00

What is the recommended speed for machining with a .500 diameter HSS two-fluted end mill?

1600 RPM*

1800 RPM

2000 RPM

2200 RPM

points:1.00

Which of the following is a primary safeguarding method for milling machines?

Using splash shields and chip barriers

Installing awareness devices around the milling table

Ensuring proper guard installation at the point of operation*

Providing safety training for machine operators

points:1.00

What safety equipment is mandatory for operators while operating milling machines?

Long hair tied back and restrained

Jewelry and loose-fitting clothing

Safety glasses and safety toe shoes*

Gloves and aprons

points:1.00

What should operators do before inspecting the part or making any adjustments on a milling machine?

Securely clamp the vise to the table

Remove the vise handle and return it to the tool box

Engage the machine way oil pump

Shut off the machine*

points:1.00

True or False: the image above is one of a single end cutting tool.

True*

False

points:1.00

Which of the following is the image above a showcase of?

Countersink*

Counterbore

Double End Cutting Tool

Four-flue end mill

points:1.00

Define hypotenuse

points:1.00

Define sine

points:1.00

Define cosine

points:1.00

Define tangent

points:1.00

Match the machining operation with its definition/description

Turning

Removing material from outside surface of part

Facing

Removing material from the end surface of part

Grooving

Cutting a recess in work

Boring

Enlarging an existing hole

Chamfering

Producing a flat, angular surface on the edge of a machined part

Knurling

Producing a raised surface of work to improve grip or appearance or to increase diameter

Treading

Forming internal or external threads with a single-point tool

Drilling

Producing a hole with a drill

Reaming

Enlarging a hole slightly with a multi-fluted cutter

Forming

Producing a desired shape by using tools to ground that shape

Shouldering

Fishing surface where two diameters or steps meet

Tapering

Producing uniform change in diameter of a part as measured along its axis

Filling

Chamfering or reducing diameter of work with a file while work is rotating

Polishing

Improving surface of part with abrasive cloth while work is turning

Cutoff

Cutting a part from work with a thin tool while work is rotating

Tapping

Producing internal threads with a tap, using the lathe tailstock to guide tap

Center Drilling

Producing a tapered hole in end of work so that part can be held between centers or be used as starting hole for drilling operation

points:17.00

Match with lathe machine part with it's corresponding letter above. A [blank], B [blank], C [blank], D [blank]

Headstock

Bed

Tailstock

Carriage

points:1.00