MAC-224 Advanced CNC Milling Final Exam Questions and Answers

What are view planes used for in CAD software?

Creating construction geometry

Storing and accessing specific views*

Setting grid parameters

Managing levels and entities

points:1.00

How can you create a new view plane in CAD software?

By selecting a grid size

By pressing the F9 button

By rotating the part using the mouse wheel*

By selecting a construction plane

points:1.00

What option allows you to snap your cursor to grid intersections in CAD software?

Z Depth

Construction planes

Grid settings*

Levels

points:1.00

What does the Z Depth feature in CAD software allow you to do?

Set the active construction plane

Select the size and spacing of the grid

Draw at a specific depth in the Z axis*

Manage levels and entities

points:1.00

How can levels be useful in CAD software?

To change the view orientation

To store and manage entities on different layers*

To set the depth for construction geometry

To adjust the grid parameters for drawing

points:1.00

What type of control is the picture above showcasing on a CNC machine?

Switch between machining or programming modes

Sub program call

Help

Split screen layout*

points:1.00

What type of control button is shown above?

Sub program call

Program management*

Switch between machining or programming modes

Soft key for selecting functions on screen

points:1.00

What machine operating mode will be activated if you pushed the button pictured above?

Manual operation

Positioning with MDI*

Program run, single block

Program run, full sequence

points:1.00

What machine operation mode will be activated by pushing the above button?

Program run, full sequence*

Manual operation

Electronic handwheel

Program run, single block

points:1.00

What feature in MasterCam allows you to add or remove a shape from selected entities?

Sweep

Loft

Fillet

Extrude*

points:1.00

Which operation in MasterCam requires selecting an axis of rotation for its function?

Fillet

Revolve*

Chamfer

Boolean

points:1.00

Which MasterCam operation is suitable for adding or removing shapes along a chain, requiring a chain for sweeping?

Fillet

Boolean

Sweep*

Loft

points:1.00

What is a characteristic of Solid Loft in MasterCam?

It requires non-coplanar and non-parallel entities.

It creates straight transitions between chains.

It can cut a body once it's created.*

It cannot be used for blending edges.

points:1.00

Which MasterCam operation is used for adding chamfers to edges or faces?

Sweep

Fillet

Loft

Chamfer*

points:1.00

Which MasterCam operation requires selecting at least 2 entities going in the same direction to avoid twisted or incorrect surfaces?

Offset

Ruled/Lofted*

Swept

Resolved

points:1.00

What is a characteristic of Revolved surfaces in MasterCam?

They require closed chains.

They must be created with at least 3 entities.

They are created by rotating chains around an axis chain.*

They cannot be rotated around splines.

points:1.00

Which MasterCam operation allows you to offset surfaces by a specified value and direction?

Net

Fence

Trim

Offset*

points:1.00

How is a swept surface created in MasterCam?

By rotating entities around an axis chain.

By offsetting surfaces along a specified direction.

By sweeping across and along chains.*

By trimming surfaces to curves.

points:1.00

What is the purpose of the TRIM operation in MasterCam for surfaces?

To create fillets between surfaces.

To trim surfaces to planes.

To extend surfaces beyond each other.

To match surface edges and trim surfaces to each other.*

points:1.00

What is a basic rule for the 5x Coordinate System in machining?

Utilize basic rotations in X/Y, X/Z, Y/Z axes.

Activate coordinate transformations like M128 / M114 during manual mode.

Move rotary or swivel axes when the machine is switched off.

Avoid basic rotations in X/Y, X/Z, Y/Z axes.*

points:1.00

What caution should be observed when using M91/M92 in the 5x Coordinate System?

M92 is set to the spindle nose as the reference point.

M92 is based on tool length activated for coordinate transformations.

M91/M92 are based on machine coordinates but do not consider tool length.*

M91/M92 are used for switching off rotary or swivel axes.

points:1.00

What action should never be taken with a machine having an active basic rotation?

Switch off the machine.

Interrupt the running program and switch to manual mode.

Move rotary or swivel axes forcefully.*

Activate coordinate transformations like M128 / M114.

points:1.00

What does G00 represent in the G-code list?

Circular motion, clockwise

Straight movement

Linear motion at rapid traverse*

Dwell time

points:1.00

Which code should be used for basic rotation over a rotary table?

G410

G403*

G201

G00

points:1.00

What does M06 indicate in the M-code list?

Tool change/Stop program run (depending on MPs)/Spindle stop*

Optional Program-Run Interruption

Constant contouring speed of tool cutting edge on arcs (increase and decrease feed rate)

Coolant ON

points:1.00