True or False: it is recommended to remove the electrode from holder when not in use

True*

False

points:1.00

How many feet should oxygen cylinders be stored away from fuel cylinders?

- 10 Feet

- 15 Feet

- 20 Feet

- 25 Feet*

points:1.00

Which of the following are safety problems in oxy-fuel equipment?

Handling high pressure cylinders

- Withdrawal rate from acetylene cylinders

- Reverse flow

- Backfire

- Flashback

- All of the above*

points:1.00

Which of the following are ways to prevent secondary voltage shock? You can select multiple answers.

- Wearing wet gloves

- Keep your welding cable and electrode holder in good condition*

- Touching the electrode

- Keep dry insulation between your body and the metal being welded*

points:1.00

Which type of fumes can cause metal fume fever?

- Barium

- Zinc*

- Silica

- Copper

points:1.00

True or False: Arc rays are incapable of burning eyes and skin?

True

False*

points:1.00

Which of the following is not a cause of backfire?

- Delivery pressure too high for the tip used*

- Loose hose connections

- Touching the tip against the work piece

- Too short of a tip-to-work distance

points:1.00

Which of the following are uses of a measuring scale?

- Measuring

- Straight edge for cutting

- Straight edge for drawing straight lines

- All of the above are uses*

points:1.00

What is the reading of the above dial caliper?

- 6.521

- 5.621*

- 5.400

- 5.892

points:1.00

Ferrous metals are alloys that contain iron.

True*

False

points:1.00

What alloying element is known for improving corrosion-resisting qualities?

- Copper*

- Cobalt

- Carbon

- Nickel

points:1.00

True or False: Solder is a soft metal alloy generally made from a mixture of copper and aluminum

True

False*

points:1.00

Which of the following is not true about backfires?

- It is momentary retrogression of the flame in the torch tip

- The user hears a “pop” and the flame is extinguished

- The flame may be reignited if an adjacent ignition source is present

- It is a major safety concern*

points:1.00

True or False: sustained backfire can cause severe damage to the torch?

True*

False

points:1.00

What is another name for SMAW?

- Stick Welding*

- Stuck Welding

- Seem Welding

- Strike Welding

points:1.00

<p>Which of the following are common uses of SMAW?

- Industrial fabrication

- Shipbuilding

- Pipelines

- Buildings

- All of the above*

points:1.00

True or False: Voltage is the movement of charge particles in a specific direction

True

False*

points:1.00

True or False: Current is measured in amps

True*

False

points:1.00

What are the parts of a SMAW outfit?

- Welding machine

- Electrode

- Workpiece leads

- Electrode holder

- All of the above are parts of a SMAW outfit*

points:1.00

Which tool is not used to clean a weld?

- Chipping hammer

- Wire brush

- Stringer brush on the grinder

- Chipping goggles

- Vacuum cleaner*

points:1.00

True or False: FCAW is used on stainless steels?

True*

False

points:1.00

Which of the following are disadvantages of FCAW?

- Irregular wire feed

- More costly filler material/wire as composed to GMAW

- Creates more fumes than stick welding

- All of the above are disadvantages*

points:1.00

Which of the following is not a piece of PPE for FCAW?

- Leather welding gloves

- Cotton welding gloves*

- Welding shield

- Leather welding jacket

points:1.00

Which of the following is not a type of weave technique?

- Crescent

- Zig zag

- Stitch

- Cross stitch*

- Box weave

points:1.00

Which of the following is a type of weld pass?

- Over pass

- Full pass

- Root pass*

- Cool pass

points:1.00

Which of the following are causes of Arc Wander?

- Current too low

- Electrode or work contamination

- Magnetic arc blow

- Unstable gas shield

- All of the above are causes*

points:1.00

Which of the following is not a disadvantage of GTAW?

- Lower deposition rates

- Requires high level of operator skill

- Lots of spatter*

- Higher level of UV rays

points:1.00

Which of the following is not a type of GTAW torch?

- Gas cooled

- Coolant cooled*

- Water cooled

points:1.00

What welding defect/discontinuity is pictured in the image above?

- Undercut

- Porosity*

- Overlap

- Excessive concavity

points:1.00

What type of welding defect/discontinuity is shown above?

- Spatter

- Excessive weld reinforcement

- Excessive convexity*

- Overlap

points:1.00

What type of weld defect/discontinuity is shown above?

- Underfill*

- Overlap

- Incomplete penetration

- Excessive penetration

points:1.00

Which of the following is not a type of porosity?

- Uniformly scattered

- Randomly scattered*

- Clustered porosity

- Linear porosity

- Piping/wormhole porosity

points:1.00

Match the caliper part with he number on the image above.

- Outside Jaws

- Inside Jaws

- Dial

- Blade

- Slider

- Reference Edge

- Clamp Screw

- Bezel Lock

- Depth Probe

- Main Scale

- Thumbwheel

points:1.00

Define what flashback is.

points:1.00

Explain the differences between semi-precision and precision measurement

points:1.00

Describe the process of quenching.

points:1.00

Describe the difference between soldering and welding.

points:1.00

What is an inert gas?

points:1.00

Describe the spray transfer process?

points:1.00

Describe the globular transfer process?

points:1.00

Describe the pulsed-arc transfer process?

points:1.00

What does SMAW stand for?

points:1.00

What does FCAW stand for?

points:1.00

What is arc wander?

points:1.00

What does GTAW stand for?

points:1.00

Define what undercut is.

points:1.00

Outline one cause for poor fusion.

points:1.00

Describe carbon arc cutting

points:1.00