# 1.1

True or False: Safety spectacles can protect eye sockets and the facial area immediately surrounding the eyes

True

False*

points:1.00

# 1.2

Which answer below is the above micrometer showing?

- 0.200*

- 0.220

- 0.205

- 0.210

points:1.00

What answer below is the example above showing?

- 0.275

- 0.215

- 0.250*

- 0.275

points:1.00

What answer below is the above example showing?

- 0.250

- 0.260

- 0.225

- 0.275*

points:1.00

# 1.4

True or False: Frequent lubrication is one of the most important steps in maintaining machine tools?

True*

False

points:1.00

Which of the following is not a vital function of lubricants?

- Cooling moving parts

- Minimize friction between moving parts

- Keeping parts looking shiny*

- Preventing rust from forming

points:1.00

True or False: Dry, cracked, or frayed belts should be replaced

True*

False

points:1.00

True or False: Cutting fluid may also take the form of a paste or gel.

True*

False

points:1.00

True or False: sump oil is unwanted oil that has mixed with cutting fluid.

True*

False

points:1.00

Which type of milling is mainly used on a manual mill?

- Convectional*

- Climb

points:1.00

What is the purpose of cutting fluid?

points:1.00

What is a refractometer?

points:1.00

# 1.5

intro: Match the Basic Lathe Operation to it's description

Turning

Removing material from outside surface of part.

Facing

Removing material from end surface of part.

Grooving

Cutting a recess in work.

Boring

Enlarging an existing hole.

Chamfering

Producing a flat, angular surface on the edge of a machined part.

Knurling

Producing a raised surface of work to improve grip or appearance or to increase diameter.

Threading

Forming internal or external threads with a single-point tool.

Drilling

Producing a hole with a drill.

Reaming

Enlarging a hole slightly with a multi-fluted cutter.

Forming

Producing a desired shape by using a tool ground to that shape.

Shouldering

Finishing surface where two diameters or steps meet.

Tapering

Producing uniform change in diameter of a part as measured along its axis.

Filling

Chamfering or reducing diameter of work with a file while work is rotating.

Polishing

Improving surface of part with abrasive cloth while work is turning.

Cutoff

Cutting a part from work with a thin tool while work is rotating. Cutoff operations should never be done on work that is held between centers or supported with a center.

Tapping

Producing internal threads with a tap, using the lathe tailstock to guide tap. Power is off when machinist performs this operation, and lathe is set in low range to prevent spindle from turning.

Center Drilling

Producing a tapered hole in end of work so that part can be held between centers or be used as starting hole for drilling operation.

points:1.00