Which of the following are job responsibilities for a CNC Operator (select all that apply

- Loading/Unloading Parts*

- Assembling the CNC machinery

- Inspection/Reporting*

- Replacing worn tools*

points:1.00

True or False: Swing arm type ATC changes tools much faster than the carousel-type ATC?

True*

False

points:1.00

True or False: G-codes are miscellaneous functions?

True

False*

points:1.00

Which of the following are applications of CNC machining? (select all that apply)

- Milling*

- Drilling*

- Boring*

- Grinding*

points:1.00

True or False: a yellow triangle safety decal describes a hazard?

True*

False

points:1.00

Which of the following are types of safety decals? (select all that apply)

- Yellow triangles*

- Blue circles

- Orange triangles

- Black circles with slash-through

points:1.00

What is tangent in trigonometry?

- Opposite over hypothenuse

- Adjacent over hypotenuse

- Opposite over adjacent*

points:1.00

True or False: for G codes, the universal rule is that codes from the same group can be used more than once on the same line?

True

False*

points:1.00

True or False: modal G codes do not remain effective until another G code from the same group is used.

True

False*

points:1.00

What G Code would you use to cancel a canned cycle?

- G98

- G101

- G80*

- G99

points:1.00

Programs must begin and end with which of the following signs?

- @

- *

- %*

- #

points:1.00

True or False: when starting a program, the only thing on that line should be the starting sign?

True*

False

points:1.00

True or False: once selected, a canned cycle is active until cancelled with a G80 code?

True*

False

points:1.00

Which of the following is not an inspection method for machine parts?

- Visual inspection

- Dimension inspection

- Material verification

- Touch inspection*

points:1.00

intro: Match the quadrant with its positioning on a coordinate plane.

Quadrant 1

Top right at X+, Z+

Quadrant 2

Top left at X+, Z-

Quadrant 3

Bottom left at X-, Z-

Quadrant 4

Bottom right at X-, Z+

points:4.00

Label each side of the triangle. A [A], B [B], C [C]

Hypotenuse

Opposite

Adjacent

points:1.00

Explain the difference between cartesian coordinate and polar coordinate systems.

points:1.00

<p>Describe the difference between conventional and climb milling?</p>

points:1.00

List five factors that can affect the accuracy of CNC machines.

points:1.00

Calculate the length of the side AB of this triangle

points:1.00

Calculate the angle in this triangle

points:1.00

What is a canned cycle?

points:1.00