What manufacturing process is the tool above used for?

- Reaming

- Drilling*

- Boring*

- Counter sinking

points:1.00

What manufacturing process is the above tool used for?

- Counter Drill

- Counter Bore

- Reaming

- Counter Sink*

points:1.00

What type of knurl is shown in the image above?

- Straight

- Diamond*

points:1.00

True or False: 3D models are the most common way of representing blueprint drawings.

True

False*

points:1.00

True or False: bilateral tolerance allows a dimension to vary both above and below basic size.

True*

False

points:1.00

True or False: the minor diameter is the smallest diameter of a thread.

True*

False

points:1.00

When it comes to pipe threads, what does NPT stand for.

- National Pipe Thread*

- Natural Pipe Thread

- Neutral Pipe Thread

- Nautical Pipe Thread

points:1.00

How should you select the grinding wheel for a surface grinder based on the material being ground?

- The harder the material, the softer the grade of the wheel should be.

- Choose a wheel with a grit size that matches the desired finish.

- Use a softer abrasive for softer materials and a harder abrasive for harder materials.*

- Only vitrified bonded wheels should be used regardless of the material.

points:1.00

What safety precaution should be taken regarding long hair when using the surface grinder?

- It should be tied back to prevent it from getting caught in the machine.*

- It should be kept untied for better visibility.

- It should be tucked into the shirt collar.

- It should be left loose to avoid restriction of movement.

points:1.00

What are some of the considerations for geometric dimensioning and tolerancing mentioned in your text?

- Diameter and thickness

- Surface finish and roughness

- True position, flatness, and concentricity*

- Length and width

points:1.00

What is the purpose of indicating the vise?

- To locate the edges of the vise

- To check the hardness of the vise

- To measure the dimensions of the vise

- To ensure the vise is properly aligned with the mill table*

points:1.00

What tool is used for tramming the head?

- End mill

- Dial indicator*

- Protractor

- Edge finder

points:1.00

You are cutting 5/16-18-3A threads. What is the approximate in-feed?&nbsp;

points:1.00

What is the angle, set from vertical, of the compound rest for threading?

points:1.00

What specific thread feature are you measuring if you are using a thread mic?

points:1.00

What is the difference between "pitch" and "pitch ø"?

points:1.00

List five safety guidelines to observe when operating the vertical milling machine.

points:1.00

Define what climb milling is.

points:1.00

Describe what tramming is.

points:1.00

Define what pitch diameter is.

points:1.00

The distance of actual contact of two mating threads measured along the length of the two threads is the what?

points:1.00

What feature of mating threads determines the class of fit?

points:1.00

How many classes of fit are there in the Unified National System?

points:1.00

What is the name for the rotating device that transmits power to the carriage for thread-cutting motion?

points:1.00

Name the device used to track the timing of half-nut lever engagement.

points:1.00

What measuring tool is used to visually inspect the form, or shape, of an external thread?

points:1.00

List three safety precautions to observe when operating a vertical bandsaw.

points:1.00

Name the three different types of tooth patterns for saws.

points:1.00

How many saw teeth should be engaged in the workpiece at any given time?

points:1.00

Explain how to use a push stick.

points:1.00

When wheel is finer, a 60 grit or a 36 grit?

points:1.00

How is a ring test performed?

points:1.00

When should a grinding wheel be dressed?

points:1.00

What semi-precision tool is used to take angular measurements?

points:1.00

Using the print, provide the X axis moves. Roughing endmill is ø 7/8, leaving 0.020; finish endmill is ø 1.125. Provide answers to 4 decimal places and do not include units. Answer the following

- Rough Pass Left Side

- Rough Pass Right Side

- Finish Pass Left Side

- Finish Pass Right Side

points:1.00