In the example of Word Address "N01 G02 X1500 Y2000 F4 S400 T10," what does the letter 'G' represent?

- Gear ratio

- Preparatory command

- Graphical representation

- General instruction

points:1.00

What is the primary difference between vertical milling and horizontal milling in CNC machining?

- The type of material removed

- The direction of tooling rotation *

- The cost-effectiveness

- The number of axes

points:1.00

In CNC machining, what distinguishes a 5-axis milling machine from a 3-axis or 4-axis machine?

- Pivoting action at the joint of the table*

- Geneva Mechanism for continuous machining

- Use of CAD/CAM programming

- Ability to simultaneously hit five different sides of a part

points:1.00

part of a CNC milling machine is responsible for holding and driving the cutting tools, as well as serving as a placeholder for various tools?

- Column and Base

- Power Feed Mechanism

- Work Table

- Spindle *

points:1.00

According to OSHA, what are the common hazards associated with points of operation?

- Electrical shocks and burns

- Pinch points and cutting edges*

- Noise pollution

- Slip and fall accidents

points:1.00

What is the function of Interlocking Guards in machine safety?

- To provide additional power to the machine

- To regulate temperature in the workplace

- To shut off power and prevent machine cycling when the guard is opened*

- To improve the aesthetics of the machine

points:1.00

In CNC programming, what does the G90 code signify?

- Canned cycle CANCEL

- Absolute Programming Positioning*

- Peck Drill - canned cycle

- Work Coordinate #1 (Part zero offset location) Return

points:1.00

What is Cutting Speed (CS) in milling, and how is it measured?

- The rate at which the tool is moved into the part, measured in inches per minute.

- The revolutions per minute (RPM) of the milling cutter.

- The distance the cutter would roll on the floor in one minute, measured in surface feet per minute (SFM).*

- The width of cut (W) in inches.

points:1.00

In milling, what is the formula to calculate Spindle Speed (RPM), and how is it applied?

- RPM = (3.82 × CS) / D

- RPM = (CS × 4) / D

- RPM = IPM × N × FTP*

- RPM = (D × RPM) / (3.82)

points:1.00

What is the relationship between cutting speed and work material hardness, and how does it influence recommended cutting speed?

- The harder the work material, the faster the cutting speed; the softer the material, the slower the cutting speed.*

- The softer the work material, the faster the cutting speed; the harder the material, the slower the cutting speed.

- Cutting speed is not influenced by work material hardness.

- Cutting speed is solely determined by the type of cutter used.

points:1.00

What is the purpose of using oil during the drilling and tapping process?

- To make the cut smoother and quieter.

- To increase the speed of the spindle.

- To keep the bits from overheating and breaking, and to clean out the chips.*

- To prevent the need for pecking.

points:1.00

What is the significance of pecking in drilling or tapping procedures?

- To rotate the handle a full turn then back a half turn.

- To ensure the drill or tap is straight and centered.

- To remove chips and allow the piece to cool, preventing overheating and breakage.*

- To select the appropriate drill size from the chart.

points:1.00

In CNC programming, what is the significance of the "/" symbol?

- It specifies the end of a CNC program.

- It defines an optional block that can be ignored with the "Block Delete" button.*

- It indicates the start of a new block.

- It is used to separate G and M codes

points:1.00

What is the correct order of blocks in a CNC program?

- Rapid to position above part, Coolant On, Spindle On, Machining operation.

- Start Program, Load Tool, Spindle Off, Coolant Off.

- Program Start, Load Tool, Coolant On, Move to safe position.*

- Spindle On, Coolant On, Rapid to safe plane, End program.

points:1.00

What is the purpose of the "%" symbol in a CNC program?

- Indicates the start or end of the program.*

- Represents a comment to the operator.

- Initiates a rapid motion.

- Defines an optional block.

points:1.00

Which G-code is used for rapid motion?

- G01

- G02

- G00 *

- G03

points:1.00

What does G43 do in CNC programming?

- Activates tool length compensation. *

- Defines an arc in the clockwise direction.

- Controls spindle direction.

- Cancels cutter diameter compensation.

points:1.00

Which G-code cancels cutter diameter compensation?

- G42

- G40 *

- G43

- G41

points:1.00

When is Cutter Radius Compensation (CRC) used in a CNC machining program?

- Only for small-sized tools

- To compensate for the tool centerline

- To shift the cutter edge along the tool path*

- Only when the tool diameter is constant

points:1.00

What is the primary advantage of using Cutter Radius Compensation (CRC) in CNC programming?

- Simplifies the CNC simulator

- Only works for contour shapes

- Reduces the need for CNC machine setup

- Allows various tool sizes without changing the part shape*

points:1.00

What does the term "center cutting" refer to in the context of endmills?

- The ability to cut on both the bottom and sides of the cutter

- The number of flutes on the endmill

- The material used to manufacture the endmill

- The capacity to create holes in a material by plunging*

points:1.00

When using jobbers drills for deep holes, what cycle is recommended to break chips?

- Continuous drilling

- Peck drill cycle*

- Spot drilling

- Center drilling

points:1.00

What is the purpose of center drilling before performing a drilling operation?

- To calculate the Z depth accurately

- To reduce burrs on parts

- To ensure a smooth chamfer

- To get the drilling operation off to a good start*

points:1.00

Which conditional switch is used to perform a test of a CNC program with no material, allowing for machine movement without actual cutting?

- Single Block

- Option Stop

- Block Delete

- Dry Run*

points:1.00

Which dimension of quality is concerned with the product's reputation and the perceived standing of the company or product in the eyes of the consumers?

- Performance

- Reliability

- Features

- Perceived Quality*

points:1.00

Which dimension of quality is concerned with how long the project with last?

- Performance

- Reliability

- Durability*

- Serviceability

points:1.00

Describe the difference between incremental and absolute position.

points:1.00

How do Fixture Sub-Plates (Tooling Plates) contribute to efficient CNC machining, especially in terms of setup time?

points:1.00

What is the main purpose of CNC interpolation in machining on CNC machines?

points:1.00

Describe the difference between Climb vs Conventional Milling

points:1.00

Fill out the final exercise code as depicted above using the above shop print.