# 1.1

What is the C-axis in CNC machining?

- The rotation of the workpiece around the X-axis

- The rotation of the workpiece around the Y-axis

- The rotation of the workpiece around the Z-axis*

- The linear movement of the cutting tool along the workpiece

points:1.00

Which statement accurately describes live tooling in CNC lathes?

- Live tooling refers to the ability of a lathe to rotate the workpiece.

- Live tooling allows the cutting tool to rotate, similar to a mill.*

- Live tooling is primarily used for drilling operations in lathes.

- Live tooling is fixed and cannot be adjusted during machining.

points:1.00

What is the purpose of the C-zero position in C-axis programming?

- It aligns with physical features of the chuck head or collet puller.

- It indicates the starting position for the X-axis movement.

- It defines the home position of the C-axis.*

- It determines the speed of the spindle rotation.

points:1.00

Which code is used to activate the C-axis machining mode?

- M45*

- M13

- G28

- G80

points:1.00

What is the main challenge when milling with live tooling on a C-axis?

- Calculating the feed rates for different axes*

- Adjusting the spindle speed for milling operations

- Ensuring the alignment of the workpiece with the C-axis

- Managing the coolant flow during milling operations

points:1.00

What is the primary direction of the Y-axis on a CNC lathe?

- Positive Y is down and away from the operator.

- Positive Y is up toward the operator.*

- Negative Y is down and away from the operator.

- Negative Y is up toward the operator.

points:1.00

What is the purpose of having a Y-axis on a CNC live tool lathe?

- To increase the X-axis travel range.

- To add extra capability for off-center machining.*

- To synchronize tooling operations.

- To reduce the number of available tools.

points:1.00

How is the Y-axis movement coordinated in a CNC lathe with simulated axis design?

- It is controlled by a single servo slide.

- It is controlled by the X-axis slide only.

- It is coordinated by two separate servo slides.*

- It is manually adjusted by the operator.

points:1.00

What is the function of the Y-axis offset register in programming a CNC lathe with a Y-axis?

- It sets the spindle speed.

- It controls the tool path.

- It adjusts the tool holder angle.

- It specifies the Y-axis position relative to the centerline.*

points:1.00

What machining process becomes available with the Y-axis on a lathe?

- Gear hobbing*

- Face milling

- Thread cutting

- Centerline drilling

points:1.00

# 1.2

The G14 code will mirror everything programed to run on the main spindle, to run on the sub-spindle?

True*

False

points:1.00

Which G code is responsible for the main-spindle?

- G14

- G15*

- G17

- G20

points:1.00

The sub spindle operates on which axis?

- Y-Axis

- X-Axis

- B-Axis*

- C-Axis

points:1.00

What are the two main advantages of dual spindle machining?

- Faster tool changes and reduced setup time.

- Enhanced surface finish and increased tool life.

- Transfer parts between spindles and machine long parts with added support.*

- Higher spindle speeds and improved chip evacuation.

points:1.00

In a dual spindle lathe, which spindle is typically considered the main spindle?

- The left spindle.*

- The right spindle.

- The sub spindle.

- The counter spindle.

points:1.00

What is the purpose of setting an offset for the B-axis in dual spindle machining?

- To control spindle speed.

- To synchronize spindle rotation.

- To locate the part accurately.*

- To adjust tool clearances.

points:1.00

Which method is used for transferring pre-sawed blanks or castings in dual spindle machining?

- Hand off method.*

- Bar Pull method.

- Cut off method.

- Torque sensing method.

points:1.00

Why is synchronizing the spindles important in dual spindle machining?

- To reduce programming complexity.

- To improve surface finish.

- To prevent tool collisions.*

- To synchronize tool changes.

points:1.00

What are wait codes used for in dual spindle machining programming?

- To synchronize spindle speeds.

- To pause one program until another reaches a specific point.*

- To adjust tool offsets.

- To calibrate work coordinates.

points:1.00