What does the C-axis of a CNC machine refer to?

- Linear movement along the X-axis

- Linear movement along the Z-axis

- Rotation of the workpiece around the Z-axis on a lathe*

- Rotation of the tool around the X-axis

points:1.00

What is live tooling in CNC machining?

- Tools that are manually operated by the machinist

- Tools that are automatically changed by the CNC machine

- Tools used for measuring workpiece dimensions

- Tools capable of rotating and machining like a mill*

points:1.00

Where are live tooling positions commonly found in swiss screw machines?

- In the spindle

- In the turret*

- In the gearbox

- In the chuck

points:1.00

What are the two most common styles of live tooling heads?

- Axial and latitudina

- Radial and angular

- Horizontal and vertical

- Axial and radial*

points:1.00

What is the purpose of activating the C-axis machining mode with code M45?

- To start the spindle

- To activate live tooling

- To connect the C-axis for rotation*

- To cancel the C-axis movement

points:1.00

How is the C-axis home position typically programmed?

- With code G28*

- With code M46

- With code M45

- With code G99

points:1.00

What machine mode change is possible for C-axis programming?

- G98 to G99*

- G99 to G98

- G28 to G29

- G29 to G28

points:1.00

Which mode is used to program the C-axis as though it is a linear axis?

- G12.1 Polar Coordinate Interpolation*

- G07.1 Cylindrical Interpolation

- G68.1 Polar Coordinate Shift

- G07.1 Polar Coordinate Shift

points:1.00

What is the primary use of the Y-axis in CNC lathe machining?

- Engraving on the workpiece

- Turning parts off centerline of the spindle*

- Drilling axial holes

- Tapping on the face of the workpiece

points:1.00

How is the Y-axis offset value typically called in CNC lathe programming?

- With code T*

- With code G28

- With code G53

- With code G99

points:1.00

What are two main advantages of dual spindle machining mentioned in the text?

- Increased tooling options and reduced programming complexity

- Simultaneous machining of multiple parts and decreased machine setup time

- Enhanced part transfer capabilities and improved surface finish*

- Higher spindle speeds and increased spindle torque

points:1.00

In a dual spindle lathe, which spindle is typically referred to as the main spindle?

- Sub spindle

- Right spindle

- Head 1*

- Counter spindle

points:1.00

What is the axis along which the sub spindle travels to and from the main spindle in the Samsung SL25 ASY?

- X-axis

- Y-axis

- B-axis*

- C-axis

points:1.00

What is the purpose of setting an offset for the B-axis in dual spindle machining?

- To control spindle speed

- To synchronize spindle rotation

- To adjust work coordinates*

- To measure tool lengths

points:1.00

Which method is used for transferring a workpiece from the main spindle to the sub spindle in bar work?

- Hand off method

- Bar Pull method*

- Torque sensing method

- Continuous machining method

points:1.00

What is essential for synchronizing spindles when transferring parts in dual spindle machining?

- Phase synchronization only

- Velocity synchronization only

- Tool length compensation

- Both velocity and phase synchronization*

points:1.00

What is the purpose of using wait codes in programming dual spindle machines?

- To increase spindle speed

- To synchronize turret movements

- To pause one program until another reaches a certain point*

- To adjust workpiece orientation

points:1.00

In dual spindle machining, what is a concern when planning tooling for the sub spindle?

- Clearance issues due to turret body limitations*

- Insufficient spindle power for heavy cuts

- Incompatibility with standard tool holders

- Programming complexity for tool changes

points:1.00

What is the function of a part catcher and ejector in dual spindle machines?

- To synchronize spindles during part transfer

- To remove completed parts without operator interference*

- To remove completed parts without operator interference

- To adjust workpiece coordinates during machining

points:1.00

What is phase synchronization in dual spindle machining?

- Aligning spindle speeds during transfer

- Synchronizing the movement of multiple axes

- Aligning the zero location of C-axis on both spindles

- Aligning the zero location of C-axis on both spindles*

points:1.00