ISC 112 Final Exam Questions and Answers:

1/3 of ALL falls are from roof and ladder falls.  This would make what occupation particularly dangerous?

Manufacturing

Meat packing

Mining

Construction*

points:1.00

For all effective safety programs the 3 Es should be included.  The 3 Es are:

Enforcement, Environment, Education

Education, Enforcement and Evaluation

Engineering, Education, Enforcement*

Engineering, Education and Evaluation

points:1.00

The 1960s in the US had tremendous changes to labor laws because:

The Civil Rights Act

The growing awareness of the environment.

Growing awareness of chemical exposure in the workplace.

Growing emphasis of the power-of-the-people.

All of the above.*

points:1.00

Is the following an acute reaction or a chronic reaction?  A chemical burn on the arm or hand.

Acute*

Chronic

points:1.00

Is the following example an acute or chronic hazard?   The development of lung cancer caused by exposure to smoke inhalation.

Acute

Chronic*

points:1.00

A chemical is classified as hazardous if it has an explosion, fire or health hazard on the label.

True*

False

points:1.00

The Community Right to Know Act was a direct result of:

The tragedy in Bopahl, India*

Workplace chemical accidental exposure in the 60s.

So many MSDS being developed.

points:1.00

A permit required confined space:

Contains an active hazard

Can interfere with worker ability to exit the space

Can cause injury or acute illness

Can cause death

All of the above*

points:1.00

A space not designed for continuous occupation:

Has no ventilation

Has no lighting

Does not have sufficient room to work

Does not have sufficient room to be occupied continuously

All of the above*

points:1.00

The first FEDERAL safety legislation was enacted in 1893 which required safety equipment in the workplace.  This was followed by the establishment of the US Bureau of Mines. Closely following this landmark legislation was:

Workers Compensation Laws*

Material Safety Data Sheets

Changes to the minimum wage

NIOSH

points:1.00

In NC, Worker's Compensation insurance rates are set by which of the following?

By specific types of employment, or those that have more risk pay higher rates

Premiums are based on every $100.00 of payroll. If payroll goes up, so do rates.

Premiums increase if there are injuries and decrease if there is not one.

Number of employees on the payroll.

All of the above.*

points:1.00

Material Safety Data Sheets are developed by:

The end user of the chemical

The Chemical Manufacturer*

The chemical shipping company

All of the above in collaboration

points:1.00

Federal OSHA allows the States to:

Make up their own plans

Follow Federal OSHA to the tee but make no changes

Follow FED OSHA standards as a minimum but are allowed to exceed the Fed Standards*

Both b and c are correct

points:1.00

The acronym OSHA stands for:

Occupational Safety and Health Association

Occupational Safety and Health Administration*

Occupational Safety and Health Act

feedback:This is commonly known as "The ACT"

points:1.00

OSHA is an agency of the:

Department of the Interior

National Institute of Occupational Safety and Health

World Health Organization

Department of Labor*

points:1.00

OSHA permits OSHA Inspectors to:

Enter the workplace during work hours.

Inspect and investigate all sites in the workplace

enter any workplace covered by the OSHAct

Inspect the OSHA Log

Both a and b only

All of the above*

points:1.00

OSHA Standards must be followed by:

Domestic workers who clean private residences.

Mining operations

Any employer who has one employee.*

Only companies with three or more employees.

None of the above.

points:1.00

The Personal Protective Equipment and Clothing standard requires:

That the employer perform a hazard assessment

The employer evaluate the effectiveness of the PPE program

That all employees attend training for PPE

The employer repair and replace worn or broken PPE

All of the above*

points:1.00

OSHA requires that all PPE meet or be equivalent to which standard?

AEPA, American Eye Protection Association

ANSI, American National Standards Institute*

ASEP, American Standard Educational Protocol

points:1.00

The most common reason(s) for slip, trip and fall accidents is (are):

Uneven walking surfaces.

Wet or slippery surfaces

Environmental conditions such as ice or snow.

All of the above*

points:1.00

The 60s in the US was a time of great production of mass goods.  Quality was not a concern and goods were sold all over the world, no matter the quality. During this era, industrial accidents accounted for 14,000 annual deaths. The Federal Government responded by which of the following:

Creating legislation to regulated workplaces and make employers accountable.

Development of the Occupational Safety and Health Act.

A shift of power from that of big business to public interest.

All of the above*

points:1.00

In North Carolina, which of the following Benefits and Wage Replacements would be for LIFE?

Temporary Total Disability

Temporary Partial Disability

Permanent Partial Disability

Total and Permanent Disability*

points:1.00

If a worker gets injured in NC and needs an attorney to prove his/her claim, who decides how much the attorney gets paid?

North Carolina Industrial Commission*

The insurance company that provides WC coverage.

The injured employee.

OSHA

None of the Above

points:1.00

What type of insurance is Worker's Compensation?

Liability must be established to be covered.

The same as your automobile insurance.

The only way to get compensated is to go to court.

No Fault*

None of the above

points:1.00

Which of the following are ways to provide Worker's Compensation Coverage in NC?

An insurance agent can write coverage

A member of a Self Insured Fund where similar industries pay into that fund.

Self insured, ie you can post a bond with the Department of  Insurance

All of the Above*

points:1.00

Which example is not a confined space?

A drilled shaft

Rooftop platform*

Chiller

Boiler

Sewer

points:1.00

Wearing a backpack can contribute to falls because it changes your center of gravity.

True*

False

points:1.00

True or False: Falling backwards is generally more dangerous than falling forward.

True*

False

feedback: Falling backwards generally results in more head injury than falling forward. Falling forward usually can be a limited fall by using the arms to break the fall.

points:1.00

True or False:  A trip most often results in a person falling backwards, while a slip most often results in a person falling forward.

True

feedback: Incorrect, A trip most often results in a person falling forward, while a slip most often results in the person falling backwards.

False*

points:1.00

In North Carolina, if a worker is injured by a fellow employee and files a compensation claim, he or she gets compensated by Worker's Compensation.

True*

False

points:1.00

True or False.  In setting up a ladder safely, you should follow the 1/4 rule.

True*

False

points:1.00

True or False:  You have chemicals in your home such as Bleach/Clorox that would require an MSDS or SDS if they were used in the workplace.

True*

False

points:1.00

True or False:  All employees who are exposed to a chemical MUST have access to the MSDS or SDS for that chemical.

True*

False

points:1.00

True or False.  North Carolina manages its own OSHA plan.

True*

False

points:1.00

True or False:  Power cords should never permanently cross a hallway or walkway.

True*

False

points:1.00

True or False:  All workplaces should be periodically assessed for changes in conditions, equipment or operating procedures that could affect occupational hazards.

True*

False

points:1.00

True or False:  Employers are required to train EACH employee who uses PPE.

True*

False

points:1.00

True or False:  Safety Data Sheets were redesigned in the past few years because of the proliferation of international shipping of chemicals and the need for a consistent data sheet. Prior to this, there were many formats for the SDS.

True*

False

points:1.00

Is this statement true or false?  Slips, trips and falls constitute the majority of general industry accidents leading to personal injury.

True*

False

points:1.00

True or False: Employers are responsible for keeping a written maintained copy of a lockout and tagging written program.

True*

False

points:1.00

Which of the following are alerting techniques for electric hazards?

Signs

Symbols

Tags

Barricades

Attendants

All of the above*

points:1.00

Which of the following is not a power tool safety tip?

Never carry a tool by the cord

It is ok to yank the cord to disconnect it from the receptacle*

Keep cords away from heat, oil, and sharp edges

Disconnect tools when not in use

points:1.00