Which of the following is not a typical danger in a welding shop or welding environment.

- Heat

- Sparks

- Fumes

- Hot metal

- Drowning*

- Flammable material

points:1.00

True or false: it is fine to wear pants with cuffs in a welding environment.

True

False*

points:1.00

True or False: Do not apply ointment, butter, or any other home remedies to a second- or third-degree burn.

True*

False

points:1.00

Which type of respirator provides a clean flow of breathing air from a secondary source?

- Negative-pressure respirators

- Positive-pressure respirators*

points:1.00

Welding helmets protect the eyes and face from arc rays (UV Rays), weld sparks, and weld spatter that directly hit the helmet. What should be worn in addition to a welding helmet to protect the eyes against slag chips, grinding fragments, and other hazards, such as wire wheel bristles?

- Steel-toed shoes

- Safety glasses*

- Bump cap

- Leather gloves

points:1.00

What class of hard hat is designed for electrical/utility work?

- Class G

- Class C

- Class E8

- Class V

points:1.00

True or False: You should inspect all respirators for wear and tear before and after each use.

True*

False

points:1.00

In the above safety hazard diamond, what does the 1 in the blue square mean?

- The material can be lethal

- There is no health hazard with this material

- This material can cause significant irritation*

- This material may explode at normal temperatures

points:1.00

True or False: the plasma and the arc do not pass through the constricting nozzle.

True

False*

points:1.00

Which of the following is not a danger for plasma arc cutting?

- Intense light

- Heat

- Radiation

- Flying particles

- All of the above are dangers*

points:1.00

Which is not an advantage of plasma arc cutting?

- Plasma arc cutting has the ability to cut all metals

- Plasma arc cutting equipment is less costly than other processes*

- No preheating is required

- Cutting speeds are faster than for oxyfuel cutting

points:1.00

Which of the following is not a disadvantage of plasma arc cutting?

- A source of electricity must be available

- The presence of an arc can cause safety hazards

- It uses hazardous or explosive gasses*

- Metal fumes can be a health hazard

points:1.00

True or False: the plasma and the arc do not pass through the constricting nozzle

True

False*

points:1.00

Which of the following is not a danger for PAC

- Intense light

- Heat

- Radiation

- Flying particles

- All of the above are dangers*

points:1.00

True or false: torches that use a mixing chamber are known as equal-pressure torches.

True*

False

points:1.00

Which of the following can cause a backfire during flame cutting?

- Overheating the tip

- Loose tip

- Damaged seats

- Dirt in the tip

- All of the above*

points:1.00

True or false: torches that use a mixing chamber are known as equal-pressure torches.

True*

False

points:1.00

Which of the following can cause a backfire during flame cutting?

- Overheating the tip

- Loose tip

- Damaged seats

- Dirt in the tip

- All of the above*

points:1.00

True or false: the low flame temperature restricts the use of oxyhydrogen flame to cutting, usually underwater.

True*

False

points:1.00

True or false: oxyfuel gas cutting is not useful for shape-cutting metal parts.

True

False*

points:1.00

True or false: Oxyfuel gas cutting outfits need to be purged after every cylinder change.

True*

False

points:1.00

True or false: the thinner the steel, the greater the time required to raise the temperature of the steel high enough to make clean cuts.

True*

False

points:1.00

Carbon arc cutting power supplies use DC electricity.

True*

False

points:1.00

DC Electrical circuits have how many charged ends

- 1

- 2*

- 3

- 4

points:1.00

Describe what arc flash is.

points:1.00

What is the purpose of a safety data sheet (SDS)?

points:1.00

What is kindling point/kindling temperature?

points:1.00

Describe the methods of controlling distortion when making cuts?

points:1.00

What type of gauge shows the pressure at the regulator?

points:1.00

What is hard slag?

points:1.00

Describe the methods of controlling distortion when making cuts.

points:1.00

Define what heat of combustion is.

points:1.00