MNT-110 Intro to Maintenance Procedures Quiz Questions and Answers

Module 1 Quiz Questions and Answers

What does OSHA stand for?

Occupational Safety and Health Administration*

Occupational Standards and Health Administration

Occasional Safety and Health Administration

Occupational Sanitization and Healing Administration

points:1.00

True or False: OSHA requires safety data sheets to communication hazards of chemical products.

True*

False

points:1.00

What type of hazards are discussed on a safety data sheet? (Select all that apply)

Biological Hazards*

Ergonomic Hazards*

Safety Hazards*

Chemical Hazards*

Physical Hazards*

points:1.00

True or False: Lockout/Tagout ensures the system is energized when doing maintenance and repair operation.

True

False*

points:1.00

What type of hard hat is not rated for electrical protection?

Class G

Class C*

Class E

points:1.00

What type of gloves would you wear for welding?

Cloth

Latex

Rubber

Leather*

points:1.00

Which of the following is not a fall protection safeguard?

Safety nets

Handrails

Grab handles

Wet floors*

points:1.00

What is the most common shock-related injury?

Death

Electrical burns*

Falling

Frizzy hair

points:1.00

Which of the following does not usually damage flexible cords?

Aging

Door Edges

Stables

Wind*

Window Edges

points:1.00

Match the electrical term with it's definition:

Current - the movement of electrical charge

Resistance - opposition to current flow

Voltage - a measure of electrical force

Conductors - substances, such as metals, that have little resistance to electricity

Insulators - substances, such as wood, rubber, glass, and bakelite, that have high resistance to electricity

Grounding- a conductive connection to the earth which acts as a protective measure

points:6.00

Define what a pinch point is.

points:1.00

What are the 6Ss in a 6S program?

points:1.00

Upload your word document where you listed the hazards presented in the hazard assignment here.

points:5.00

Module 2 Quiz Questions and Answers

The longer that defects are left unattended to, the more time and expense will incur once the machine is repaired.

True*

False

points:1.00

What are warning signs of a machine breaking down? (Select all that apply)

Change in vibration*

Temperature changes*

Motor current signature*

Oil condition*

points:1.00

The sound of vibration changing is not a warning sign of a machine breaking down

True

False*

points:1.00

What is the correct sequence of steps for a predictive maintenance program?

Setup &gt; Monitoring &gt; Testing &gt; Repair &gt; Scheduling Inspection

Setup &gt; Repair &gt; Scheduling Inspection &gt; Testing &gt; Monitoring

Setup &gt; Testing &gt; Monitoring &gt; Repair &gt; Scheduling Inspection*

Testing &gt; Setup &gt; Monitoring &gt; Scheduling Inspection &gt; Repair

points:1.00

Which of the following is NOT a primary use of ultrasound analysis (UA)?

Leak detection &nbsp;

Mechanical inspection &nbsp;

Employee productivity tracking&nbsp;*

Electrical inspection &nbsp;

points:1.00

What is one of the main advantages of ultrasound analysis (UA) compared to other predictive maintenance (PdM) technologies?

Higher data accuracy &nbsp;

Lower acquisition costs and ease of use&nbsp;*

Ability to detect software issues &nbsp;

Longer equipment lifespan &nbsp;

points:1.00

Which of the following is a primary method used in oil analysis for predictive maintenance?

Visual inspection for contamination and viscosity &nbsp;*

Monitoring employee performance&nbsp;

Analyzing software issues &nbsp;

Testing fuel efficiency &nbsp;

points:1.00

What is one of the benefits of conducting oil analysis as part of a predictive maintenance strategy?

Reducing the need for visual inspections &nbsp;

Early detection of unexpected equipment wear &nbsp;*

Increasing the frequency of oil changes &nbsp;

Improving software performance &nbsp;

points:1.00

Which of the following is a practical task that thermal imaging helps accomplish in preventative maintenance?

Enhancing software performance&nbsp;

Detecting early signs of equipment failure or malfunctions*

Monitoring employee productivity &nbsp;

Reducing material costs &nbsp;

points:1.00

What is one of the main benefits of using thermal imaging in a preventative maintenance program?

Identifying abnormal operating behavior quickly&nbsp;*

Increasing the frequency of oil changes

Enhancing visual inspection capabilities &nbsp;

Improving water content measurement &nbsp;

points:1.00

Which of the following is NOT typically included in a work order letter?

Company name

Work order title &nbsp;

Employee salary details&nbsp;*

Work location &nbsp;

points:1.00

True or False: if gloves are worn during work, an employer must make sure that the hand tools being used can still be gripped easily without risk of slipping?

True*

False

points:1.00

<p>Which of the following is not a common injury from a hand tool:</p>

<p>Severe cuts</p>

<p>Punctures that require stitches</p>

<p>Eye injuries/blindness</p>

<p>Carpal tunnel syndrome</p>

<p>Scrapes and abrasions</p>

Amputations*

points:1.00

What are things to take into consideration when selecting a hand tool? (Select all that apply)&nbsp;

It is designed for the task*

It fits your hand size*

Keeps your wrist straight*

<p>Fits in the workspace available</p>*

<p>Can be used in a comfortable work position</p>*

<p>Requires minimum use of force</p>*

points:1.00

<p>True or False: always inspect hand tools before using them?</p>

True*

False

points:1.00

<p>True or False: it is ok to use hand tools that are rusted.</p>

True

False*

points:1.00

<p>What tool can be used as an alternative to a wrench?</p>

<p>Hex Keys</p>

Sockets*

<p>Screwdrivers</p>

<p>Pliers</p>

points:1.00

<p>What type of wrench is used with sockets?</p>

Torque*

<p>Phillips head</p>

<p>Crescent</p>

<p>Pipe</p>

points:1.00

<p>True or False: every screwdriver with a plastic handle is properly insulated.</p>

True

False*

points:1.00

<p>Which of the following are types of pliers?</p>

Slip-joint&nbsp;*

Lineman's*

<p>Phillips head</p>

<p>Racket</p>

points:1.00

<p>Which of the following is not a type of hammer?</p>

Claw

Ball-peen

Lock jaw*

Brass

Dead-blow

points:1.00

<p>True or False: when working with drills, be sure to unplug it before changing bits</p>

True*

False

points:1.00

<p>Which type of saw is most often used to cut wood?</p>

Circular*

Reciprocating

points:1.00

When using a 3/4th inch hole saw and cutting on aluminum material, what is the RPM you should use?

1800*

690

270

240

points:1.00

True or False: as a general rule, the harder or more brittle the material, the slower you should cut.

True*

False

points:1.00

When using a 3/4th inch hole saw and cutting on ceramic material, what is the RPM you should use?

430

500*

370

210

points:1.00

Explain the difference between proactive maintenance and reactive maintenance.

points:1.00

Upload your preventative maintenance assignment here.

points:5.00

Module 3 Quiz Questions and Answers

What letter designation of blueprint is 8.5x11 in. in size?

A*

B

C

D

points:1.00

True or False: on a title block, scale is a inverse relationship between actual size and size shown.

True

False*

points:1.00

How many views can there by on one drawing?

Three

Four

Five

Six*

points:1.00

True or False: maximum material condition is the size condition containing greatest amount of material within tolerance limits?

True*

False

points:1.00

True or False: each wiring diagram should have a legend/abbreviation page.

True*

False

points:1.00

Match the type of diagram with its definition.

Schematic Diagrams - Shows electrical connections and operation of circuit without showing exact location of each component

Ladder diagram - Shows function of a circuit and resemble a ladder.

Wiring diagrams - Shows arrangement and location of electrical components in a circuit.

What is the purpose of phantom lines?

points:1.00

What is an assembly drawing and what is it's purpose?

points:1.00points:4.00

Module 4 Questions and Answers

Which of the following is not a main thread series in Unified system?

Unified Coarse

Unified Fine

Unified Extra Fine

Unified High Pitch*

points:1.00

True or False unified fine (UNF) series has smaller thread depth than coarse threads.

True*

False

points:1.00

Which of the following thread classes has the greatest tolerance?

2A

2B

1A*

3B

points:1.00

True or False: Bolts are internally threaded fasteners screwed to a nut.

True

False*

points:1.00

Which of the following is not a fastener head type?

Flat Head

Oval Head

Hex Head

Pentagon Head*

points:1.00

Which of the following is not a type of nut?

Hexagonal

Jarn

Jam*

Tee

points:1.00

What is one of the tools you can use to tighten a nut and bolt by hand?

Electric drill

Pliers

Ratchet wrench*

Screwdriver

points:1.00

Which method is recommended to remove a stuck bolt if lubricant and leverage fail?

Heating the bolt *

Using an adjustable wrench

Stabilizing the bolt with pliers

Using a screwdriver

points:1.00

True or False: taper pins position parts and lock them together.

True*

False

points:1.00

Explain the difference between dies and taps.

points:1.00

Upload your maintenance procedure here.

points:5.00

Module 5 Quiz Questions and Answers

Which of the following is a not tool typically used for smaller lifts?

Pry bar

Manual jack

Hydraulic jack

Dolly

Chains*

points:1.00

Which of the following is not a common rope material?

Manila

Nylon

Leather*

Polypropylene

Polyester

points:1.00

According to federal regulations, rope used for rigging must have a safety factor of what?

5:1*

4:1

1:1

3:1

points:1.00

What is creep in rope safety?

Tendency of rope to stretch when load is applied

Tendency of rope to turn when load is applied

Permanent separation of fibers over time*

points:1.00

Every time you use rope you should inspect it for (select all that apply):

Stretch*

Thin areas*

Abrasions*

Chemical exposure*

points:1.00

Every time you use a sling you should inspect it for (select all that apply):

Deformation*

Damage*

Frays*

Kinks*

points:1.00

OSHA mandates sling angles be how many degrees or greater?

50

30*

40

20

points:1.00

True or False: shackles can be used to connect to chains to hooks?

True*

False

points:1.00

What situation would you use shackles for rather than using an eyebolt?

points:1.00

What situation would you use a chain rather than a robe when rigging something?

points:1.00

Module 6 Quiz Questions and Answers

Kinetic energy is measured by what?

Watts

Grams

Joules*

Meters per second

points:1.00

True or False: Potential energy is the stored energy of an object?

True*

False

points:1.00

What is a measure of force applied to unit area?

Torque

Pressure*

Voltage

Work

points:1.00

What calculation is the above image for?

Pressure

Power

Torque*

Potential Energy

points:1.00

True or False: more power is required to do work in shorter time.

True*

False

points:1.00

True or False: levers are defined by the position of fulcrum, output force, and load.

True

False*

points:1.00

Screws convert ________ torque to output force.

Input*

Output

Clockwise

Counter clockwise

points:1.00

What is the definition of energy?

points:1.00

What are the three classes of levers?

points:1.00

What is the difference between static friction and kinetic friction?

points:1.00

Upload your PM schedule for the mill here.

points:1.00

Upload your lathe PM schedule here.

points:1.00

Module 8 Quiz Questions and Answers

Which of the following is not a fact about bearings?

They are mechanical components that supports moving parts

They reduce friction and restrict motion

Inside parts supports moving parts

Can be used for insulation*

points:1.00

True or False: ball bearings and roller bearings are types of rolling-element bearings.

True*

False

points:1.00

True or False: angular-contact bearings are the most common type of bearings.

True

False*

points:1.00

When removing bearings from packaging, you should:

Not clean or spin with pressurized air

Not lubricate prior to installation

Clean with pressurized air*

Not compromise cleanliness

points:1.00

What is one of the primary functions of bearing lubrication?

To increase friction between moving parts

To separate moving parts to prevent wear and tear*

To eliminate the need for maintenance

To replace the need for bearings

points:1.00

Which type of lubricant is generally used for high-speed or high-temperature applications?

Grease

Water

Oil*

Powder

points:1.00

What is the percentage of bearing applications that typically use grease as a lubricant?

10-20%

30-40%

50-60%

80-90%*

points:1.00

Which of the following is NOT a method for bearing lubrication mentioned in the text?

Drip feed lubrication

Oil splash feed lubrication

Water mist lubrication*

Force feed lubrication

points:1.00

What is a common mistake that can lead to bearing failure?

Using the correct amount of lubricant

Over- and under-lubrication*

Regularly monitoring lubrication levels

Using a compatible lubricant

points:1.00

What is the difference between a thrust bearing and a needle bearing?

points:1.00

Upload your bearing research assignment here.

points:1.00

Module 9 Quiz Questions and Answers

What material are shafts not typically available in?

Plastics

Ceramics

Steels

Clay*

points:1.00

True or false: checking for shaft runout is the easiest way to find bent or bowed shafts?

True*

False

points:1.00

Which of the following is not a factor that affects shaft tolerance?

Type of load

Type of bearing

Material of load*

Shaft diameter

points:1.00

A pillow block does what (select all that apply)

Houses bearings*

Supports rotating shafts*

Conduct electricity

Provides structural integrity to a machine

points:1.00

True or False: ball and roller bearings are self-aligning.

True

False*

points:1.00

Which of the following is not used to accomplish bearing-to-shaft fit?

Setscrews

Eccentric locking collar

Hammering*

Tapered adapter sleeve

points:1.00

What type of joint/bearing is shown in the image above?

Universal Joint*

Ball Bearing

Cylindrical Bearing

Ball and Roller Bearing

points:1.00

True or False: couplings connect two shafts.

True*

False

points:1.00

What material are flexible couplings not made of?

Metal

Plastic

Wood*

Rubber

points:1.00

What do you check a baseplate for during a prealignment check? (select all that apply)

Cracks*

Voids*

Levelness*

Debris*

points:1.00

Which of the following alignment tools is inserted between the baseplate and machine's foot to achieve level?

Dial indicator

Shims*

Friction clamps

Machinist rule

points:1.00

During initial alignment the most important aspect is to get motor and driven machine as parallel as possible?

True*

False

points:1.00

What is the purpose of a universal joint?

points:1.00

How does a gear coupling work different compared to other couplings?

points:1.00

What is soft foot?

points:1.00

Module 10 Quiz Questions and Answers

Which of the following is not an advantage of belt drives?

Less regular maintenance

Easier to replace

Can be used over a wide range of speeds

Can be over tensioned without placing excess load on bearings*

points:1.00

Which of the following is not a type of belt?

V-belt

Double-side belt

Flat belt

Full belt*

points:1.00

True or false: for a process that requires running at a higher speed, you would use a synthetic belt?

True*

False

points:1.00

What is a sheave?

A type of pulley

A type of belt

The wheel component of a pulley

A type of conductor

points:1.00

Which of the following is not a sheave type discussed in the lecture?

Single-belt sheave

Copper belt sheave*

Multiple-belt sheaves

Variable-speed sheaves

points:1.00

Which of the following are considerations when designing a drive? (select all that apply)

Application*

Speed of drive and driven machine*

Distance between pulleys*

Material the pulley made out of

points:1.00

Ture or False: you can store belts on a shelf that gets hit by direct sunlight?

True

False*

points:1.00

True or False: when storing belts, be sure to keep them away from ozone-producing machinery?

True*

False

points:1.00

Which of the following is a main method of lubrication for chains?

Oil bath*

Water flinger

Grease bath

Water bath

points:1.00

True or false: gear backlash is side-to-side clearance between mating teeth.

True*

False

points:1.00

What type of machinery would you use a flat belt for?

points:1.00

What is the difference between dynamic balance and static balance?

points:1.00

What is a leaf chain?

points:1.00

What circumstances would you use a silent chain in?

points:1.00

Module 11 Quiz Questions and Answers

What is ductility?

Ability to deform without breaking*

Resistance to penetration

Measure of stress applied

Tendency to fracture

points:1.00

What is hardness?

Tendency to fracture

Resistance to penetration*

Ability to deform without breaking

Measure of stress applied

points:1.00

In a stress-strain curve, which value is shown on the Y axis?

Stress*

Strain

points:1.00

What is resolution?

How close measurement is to standard value

How well system reproduces an established outcome

Smallest increment system can measure/recognize*

Largest increment system can measure/recognize

points:1.00

True or False: Corrosion is the deterioration of metal components.

True*

False

points:1.00

What is natural frequency in machinery vibration analysis?

Speed at which rotating element reaches resonance

Measure object vibration with damping

Measure object vibration without damping*

The output force of an object

points:1.00

True or False: it is impossible to apply too much lubrication to machine parts?

True

False*

points:1.00

Which of the following is not a signifier of improper initial installation of a machine part?

Excessive piping strain

Improperly seated bearings

Long-term alignment problems

Excessive documentation*

points:1.00

Describe how the stress-strain curve works?

points:1.00

Describe how the bathtub curve works?

points:1.00

What is the difference between adhesive wear and corrosive wear?

points:1.00

Describe the procedure of measuring shaft speed?

points:1.00

Module 12 Quiz Questions and Answers

True or False: in pneumatic systems air treatment removes moisture?

True*

False

points:1.00

In a water/oil separator, what is the air filtered through?

Carbon

Zeolite*

Sand

Pebbles

points:1.00

True or False: secondary air treatment takes place after point of use?

True

False*

points:1.00

Which of the following is not true of single-state compressors?

Draws in standard-pressure air

Discharges compressed air

Air passes through intercooler between stages*

One or more pistons driven by same shaft

points:1.00

True or False: rotary compressors are designed for constant use.

True*

False

points:1.00

True or False: you do not have to do lockout/tagout when working with motors?

True

False*

points:1.00

Which of the following are signs of compressor failure? (Select all that apply)

Excessive heat generation*

Little to no vibration

Excessive noise*

Consistent noise

points:1.00

True or False: you should check lubrication levels at least every 500 hours.

True*

False

points:1.00

How do air compressors increase air pressure?

points:1.00

Describe how a directional flow regulator works?

points:1.00

Upload your completed maintenance procedure here.

points:5.00

Module 13 Quiz Questions and Answers

Which of the following statements is true about mass and weight?

Mass is dependent on its location, while weight is independent of location.

Weight is the amount of matter of an object, while mass is the force due to gravity.

Mass is the amount of matter of an object and is independent of its location, while weight is dependent on its location.*

Weight is measured in units of mass, while mass is measured in units of weight.

points:1.00

An object weighs 1000 pounds. Given that the acceleration due to gravity is 32.2 ft/s², what is the mass of the object in slugs?

31.06 slugs*

32.2 slugs

1000 slugs

2000 slugs

points:1.00

Which of the following is true about laminar flow?

It occurs when the fluid flows in parallel layers with no disruption between the layers.*

It is also known as chaotic flow.

It involves rapid variation of pressure and velocity of the fluid.

It is a good way to transfer thermal energy.

points:1.00

How many ports does a four-way control valve have?

One

Two

Three

Four*

points:1.00

Identify the components on a schematic drawing of a pneumatic system.

A - Spring

B - Exhaust port

C - Air pressure inlet

D - Solenoid

E - Directional control valve

F - Cylinder actuator

G - Flow control valve

points:1.00

Module 14 Quiz Questions and Answers

What do pumps do in a hydraulic system?

Convert fluid energy into motion

Transfer energy into fluid/move fluid through system*

Store fluid

Control the hydraulic system

points:1.00

Which of the following is not something a reservoir removes from hydraulic fluid?

Heat

Air

Moisture

Plasma*

points:1.00

True or False: a reservoir can serve as a secondary cooling system for a hydraulic system?

True*

False

points:1.00

Which of the following is not a typical reservoir design?

Horizontal

Vertical

I-Shape*

L-Shape

points:1.00

True or False: you do not need to utilize lockout/tagout when servicing a hydraulic system?

True

False*

points:1.00

Which of the following is a sign of a possible pump problem?

Excessive noise*

Little to no particles in filter after pump

Consistent output power

Consistent discharge pressure

points:1.00

True or False: directional control valves control flow of fluid into paths?

True*

False

points:1.00

True or False: varnishing is the accumulation of fine particles in a valve?

True

False*

points:1.00

What do hydraulic cylinders do?

Convert fluid energy to liminal motion

Convert fluid energy to linear motion*

Convert fluid energy to parallel motion

Convert fluid energy to potential motion

points:1.00

Match the part of the hydraulic system with the letter above that matches it's location in a typical system.

A - Filter

B - Pump

C - Pressure Relief Valve

D - Control Valve

E - Reservoir Tank

points:1.00

What is the purpose of a sacrificial anode?

points:1.00

How does a pressure-compensated van pump work differently than other hydraulic pumps?

points:1.00