ELC-128 Quiz Questions and Answers

Module 1 Quiz Questions and Answers

True or False: Fixed I/Os come with separate removable units.

True

False*

points:1.00

Match the PLC panel part with it's correct position as indicated by the picture above.

A - 24 Volt Power Supply

B - Serial Port

C - PLC

D - Master Contactor

E - Terminal Block

F - Wireways

points:1.00

Match the part of the master control relay with it's correct position above.

A - Line Terminals

B - Coil Terminals

C - Load Terminals

points:1.00

Match the part of the processor module with it's correct position as indicated above.

A - Status Indicators

B - Mode Selector Switch

C - Ethernet Port

D - Memory Card Receptacle

E - Serial Port

points:1.00

List three advantages of PLCs over relays

points:1.00

Module 3 Quiz Questions and Answers

What two numbers are part of the binary system

1 and 2

1 and 0*

2 and 3

3 and 4

points:1.00

True or False: a logic gate is a circuit with several inputs but only one output that is activated by particular combinations of input conditions.

True*

False

points:1.00

What is the most common PLC programming language?

Hardwired Logic

Programmable Control

Ladder*

Parallel

points:1.00

Explain how the AND function works.

points:1.00

Explain how the OR function works.

points:1.00

Explain how the NOT function works.

points:1.00

Module 4 Quiz Questions and Answers

The decimal system has a base of how many numbers?

8

10*

5

2

points:1.00

True or False: in the binary number system, the only allowable digits are 0 and 1.

True*

False

points:1.00

The hexadecimal system has a base of how many numbers?

10

12

14

16*

points:1.00

Which of the following is not a part of PLC memory organization?

Words

Bytes

Triple Words*

Double Words

points:1.00

What two numbers are not allowed in the octal numbering system?

1 &amp; 2

4 &amp; 5

8 &amp; 9*

10 &amp; 11

points:1.00

Which of the numbering systems we covered this week utilizes letters?

Binary

Hexadecimal*

Octal

Decimal

points:1.00

True or False: the binary coded decimal (BCD) system uses 4 bits to represent each decimal digit.

True*

False

points:1.00

Which numbering system does a thumbwheel switch use?

Hexadecmial

Binary

Binary coded decimal*

Decimal

points:1.00

True or False: gray code is set up so that as we progress from one number to the next, two bits change?

True

False*

points:1.00

What is the BCD representation of the number 1 in a decimal system?

points:1.00

Module 5 Quiz Questions and Answers

Which of the following are not values 1 and 0 can represent in a binary system?

ON or OFF*

Open or Closed

True or False

Near or Far*

High or Low

points:1.00

True or False: a logic gate is a circuit with several inputs but only one output that is activated by particular combinations of input conditions.

True*

False

points:1.00

True or False: for an OR gate, the output is 1 only if both inputs are 1.

True

False*

points:1.00

True or False: for a NOT gate, the output is 1 if the input is 0.

True*

False

points:1.00

Which logic gate can only have one input?

AND

OR

NOT*

NAND

points:1.00

True or False: the mathematical study of the binary number system and logic is called Boolean algebra.

True*

False

points:1.00

If you want to know when matching bits in two different words are both ON you use the ______________ function.

NOR

NOT

OR

AND*

points:1.00

If you want to know when one or both matching bits in two different words are ON you use the _______________ function.

NOR

NOT

OR*

AND

points:1.00

Module 6 Quiz Questions and Answers

What type of memory addresses are derived using the rack number, the I/O module slot number and the screw terminal number where the I/O device is wired.

Rack-Based*

Tag-Based

Ladder Logic

Lock-Based

points:1.00

True or False: Data files are the part of processor memory that stores the user ladder logic program.

True

False*

points:1.00

The SLC 500 stores data in data tables that are based on how many bit words?

8 Bit

16 Bit*

32 Bit

64 Bit

points:1.00

True or False: the time it takes to complete a scan cycle is a measure of how fast the controller can react to changes in inputs.

True*

False

points:1.00

Which of the following is not a function of scan time?

The length of the ladder program

The speed of the processor module

The type of instructions executed

The length of the processor module*

points:1.00

True or False: two major categories of PLC programming languages are textural and graphical.

True*

False

points:1.00

Memory space is divided into two types of files? What are they?

points:1.00

Define what ladder logic is.

points:1.00

Module 8 Quiz Questions and Answers

True or False: address indicates what input is connected to what device.

True*

False

points:1.00

What shapes are inputs and outputs typically represented by?

Diamonds and circles

Circles and squares

Squares and diamonds*

Circles and triangles

points:1.00

True or False: parallel branches can be used to allow more than one combination of input conditions.

True*

False

points:1.00

True or False: an internal output does directly control an output field device.

True

False*

points:1.00

What mode is used to operate or monitor the user program without energizing any outputs?

Program mode

Run mode

Test mode*

Remote position

points:1.00

Module 10 Quiz Questions and Answers

True or False: the done bit is the bit set when the accumulated value reaches the preset value.

True*

False

points:1.00

True or False: mechanical timing relays are used to speed up the opening or closing of contacts for circuit control.

True

False*

points:1.00

What term compares to when one timer's output triggers another timer's input?

Hardwired

Cascaded*

Rushed

Combined

points:1.00

What is the difference between retentive timers and non-retentive timers?

points:1.00

What is the difference between preset value and accumulated value?

points:1.00

Module 11 Quiz Questions and Answers

What type of counters are the majority of counters used in industry?

Down-counters

Up-counters*

Up/down counters

Side counters

points:1.00

Up-counters are always rest to what number.

0*

2

4

6

points:1.00

Up-counters are used to count what types of transitions?

Left to right

Right to left

True to false

False to true*

points:1.00

A one-shot contact is also known as what?

A transitional contact*

A mechanical contact

A parallel contact

A decrement contact

points:1.00

In regards to status bits, what is the difference between overflow and underflow?

points:1.00

Module 12 Quiz Questions and Answers

PLCs were originally designed to replace control ____________ that made logic decisions?

Starter switches

Relays*

Contactors

Push buttons

points:1.00

What type of relay is designed to handle heavy power loads?

Motor Starters

Relays

Contactors*

Manually Operated Switches

points:1.00

True or False: mechanically operated switch is controlled automatically by factors such as pressure, position, or temperature.

True*

False

points:1.00

True or False: pressure switches are used to control the pressure of solids.

True

False*

points:1.00

True or False: Capacitive proximity sensors operate within an electrostatic field.

True*

False

points:1.00

Describe the difference between normally open (NO) and normally closed (NC) contacts.

points:1.00

Describe how electromagnetic latching relays work?

points:1.00