HYD-110 Module Quiz Questions and Answers

Which of the following is not an advantage of pneumatics over hydraulics?

Higher speeds

Cannot leak oil

Components cost lower

Are better for higher force applications*

points:1.00

Which of the following is not a basic component of pneumatic systems?

Power Input Device

Control Device

Oil*

Gas

Conductors

points:1.00

True or False: the definition of pressure is the intensity of force created when a force from one object acts over the area of another.

True*

False

points:1.00

Which of the following is not the component of a cylinder?

Piston Seals

Rod Seals

Rotary*

Rod Bushing

points:1.00

What component of a pneumatic system is the above a symbol for?

Conditioning Device

Pressure Valve

Cylinder*

Air Motor

points:1.00

What is the pneumatic component that the above image is a symbol for?

Air Motor*

Junction

Plug

Pressure Valve

points:1.00

What pneumatic component is the above image a symbol for?

Cylinder

Pressure Valve*

Variable

Flow Direction

points:1.00

What do control valves do?

points:1.00

What is the mathematical formula for measuring pressure?

points:1.00

How does a pressure gauge work?

points:1.00

Module 2 Quiz Questions and Answers

Single-acting pneumatic cylinders are powered by compressed air in how many directions?

One*

Two

Three

Four

points:1.00

In a single-acting, spring-return cylinder, the spring is there to return the rod assembly to it's de-energized position.

True*

False

points:1.00

What type of cylinder is the above schematic symbol for?

Single-Acting to Retract Cylinder

Single-Acting to Extend Cylinder*

Double-Acting to Retract Cylinder

Double-Acting to Extend Cylinder

points:1.00

True or False: you would use an air motor where high force rotary output is needed.

True

False*

points:1.00

Which of the following is not a part of an air motor?

Housing

Rotor

Shaft

Cylinder*

Vanes

points:1.00

What type of motor is shown in the image above?

Uni-Directional Air Motor

Uni-Directional Gas Motor

Bi-Directional Air Motor*

Bi-Directional Gas Motor

points:1.00

Match the cylinder options and the description for it.

Cushions - In either or both ends of the cylinder control deceleration and relieve impact at the end of the stroke, improving durability.

Bumpers - Rubber discs that absorb impact and noise at the end of stroke, reducing noise and vibration.

Stop tubes/dual pistons - Manage side loads by distributing the forces over a larger area of the cylinder.

Rob wipers - Help keep contaminants out of the cylinder by wiping the rod as it retracts

Flow controls - Help assure constant, controllable air flow for precise and consistent speed control regardless of variations in air pressure.

points:5.00

Explain the difference between normally open and normally closed set ups.

points:1.00

Explain the difference between pivot mounting and rigid mounting.

points:1.00

Explain the difference between double-acting and single-acting cylinders?

points:1.00

Module 3 Quiz Questions and Answers

In a pneumatic cylinder, the pressure will rise to a level that creates a force equal and opposite to the force created by what?

Weight

Current

Load*

Vacuum Seal

points:1.00

True or False: spring rate is the force it takes to compress a spring a given distance?

True*

False

points:1.00

When retracing a load, the pressure can be exerted all over the surface inside the rod end of the cylinder.

True

False*

points:1.00

Which of the following are the two ways that pneumatic pressure can be stated? (Select all that apply)

Absolute pressure*

True pressure

Force pressure

Gauge pressure*

points:1.00

True or False: gauge pressure indicates the amount of pressure above atmospheric pressure.

True*

False

points:1.00

What part of a pneumatic system creates the air pressure that makes the system work?

Air Gauge

Motor

Valves

Air Compressor*

points:1.00

What does the following formula measure: F = k x d

points:1.00

Explain how Pascal's Law works.

points:1.00

Module 4 Quiz Questions and Answers

An air compressor creates _________________

points:1.00

Boyle's Law states that ________________

points:1.00

Delta P is the difference in pressure between __________________

points:1.00

Pascal's Law applies only to ______________________

points:1.00

Pressure beyond that to overcome all resistive loads determines ____________

points:1.00

Two types of resistance to air flow in a pneumatic system are __________

points:1.00

To convert gauge pressure to absolute, ________________

points:1.00

Module 5 Questions and Answers

Which of the following is not a component of a needle valve?

Valve Body

Adjustment knob*

Adjustment Screw

points:1.00

What is flow most often measured by?

Weight

Pressure

Volume*

Width

points:1.00

The way to read a rotameter is to read the scale ______.

top of the ball

center of ball*

bottom of the ball

none of above

points:1.00

Two common types of check values used in pneumatics are the ______.

spool and sleeve

vane and rotor

radial and axial

ball and poppet*

points:1.00

Which of the following is not a part of a flow control value?

Body

Vane*

Adjustment screw

Check valve

points:1.00

What is the primary function of a needle valve?

points:1.00

Define flow rate.

points:1.00

Describe the differences between volume at a given pressure, free air volume, and standard air volume.

points:1.00

Module 6 Questions and Answers

The valve control element that consists of a precision hole used to control the rate of airflow is called a{n} _________.

Inlet port

Land

Fixed orifice*

Outlet port

points:1.00

What type of valve allows free flow of air through a line in only one direction?

Shutoff valve

Check valve*

Three-way valve

Four-way valves

points:1.00

What does a diaphragm do in a pneumatic system?

points:1.00

What does a pressure booster do?

points:1.00

What are exhaust mufflers used for?

points:1.00

Module 8 Quiz Questions and Answers

Which of the following components is not part of a hydraulic system?

Power input device

Control devices

Power output devices

Air*

Conductors

Liquid

points:1.00

True or False: the suction line is where the oil from the reservoir enters the hydraulic system.

True*

False

points:1.00

True or False: an singular actuator line can carry oil both to and from the actuator.

True

False*

points:1.00

Which of the following is not a component of a hydraulic power unit?

Reservoir

Hydraulic pump

Prime mover

Prime motor*

points:1.00

Which of the following components is the above schematic symbol showcasing?

Cylinder*

Blocked line

Conductor

Spring

points:1.00

Define what force is.

points:1.00

Describe how force is calculated.

points:1.00

Upload your video summaries here.

points:1.00

Module 9 Quiz Questions and Answers

Which of the following are not common types of flow meters? (select all that apply).

Rotameter

Tube flow meter*

Turbine flow meter

Barometer*

points:1.00

Which of the following is not a common type of fixed-displacement pump?

Piston pumps

Vane pumps

Rotary pumps*

Gear pumps

points:1.00

True or False: piston motors are used where high performance is needed.

True*

False

points:1.00

True or False: vane motors are best used for light-duty mobile situations.

True

False*

feedback: Gear motors are best for this type of situation

points:1.00

True or False: one basic rule when drawing hydraulic schematics is that lines are drawn either vertically or horizontally.

True*

False

points:1.00

Describe how a flow meter works.

points:1.00

Describe how a turbine flow meter works.

points:1.00

Describe the difference between variable displacement and fixed-displacement.

points:1.00

Upload your video summary/responses here.

points:1.00

Module 10 Quiz Questions and Answers

True or False: in a hydraulic cylinder, the pressure will rise to a level that creates a force which is equal and opposite to the force created by the load.

True*

False

points:1.00

True or False: hydraulic leverage makes it impossible to generate very large forces to move heavy loads with a small input force.

True

False*

points:1.00

When using hydraulic leverage, distance is ___________

Contained

Flowing

Sacrificed*

Solid

points:1.00

The energy used to overcome frictional resistance is lost in the form of _______________

Heat*

Air

Momentum

Fluid

points:1.00

Explain how Pascal's Law works.

points:1.00

Module 11 Quiz Questions and Answers

As oil is pumped through a hydraulic system, it progressively _______.

drops in flow rate

drops in pressure*

increase in flow rate

increase in pressure

points:1.00

Delta P is another name for ______.

flow rate

the flow direction arrow on a valve symbol

pressure differential*

valve

points:1.00

Flow occurs between two connected points in a hydraulic system when _____.

there is a vacuum

a pressure differential exists*

there is high pressure

pressure is the same at both points

points:1.00

Pascal's Law states that pressure in a confined fluid is equal and acts ______.

on each flat surface

perpendicular to all surfaces*

only at the bottom of the vessel

only against the external load

points:1.00

Pressure is created by the ______.

pump

reservoir

resistance to flow*

flow meter

points:1.00

Pressures are stated as _____.

PSI

PSIA

PSIG

All of the above*

points:1.00

The area of a circle can be calculated by ______.

pi times (r squared)

pi times (d squared divided by 4)

0.7854 times (d squared)

All of the above*

points:1.00

The hydraulic level provides ______.

flow multiplication

a pressure drop

power multiplication

force multiplication*

points:1.00

The rod side area that produces the pull force on a double-acting cylinder is called the ______.

blind

cap

annular*

rod

points:1.00

Module 13 Quiz Questions and Answers

True or False: a sequence valve is a relief valve with an internal drain line.

True

False*

points:1.00

True or False: sequence valves are best used for simple applications.

True*

False

points:1.00

Which of the following is not a part of a direct-operated sequence valve?

Body

Adjustment Knob

Spool

Slide*

Spring

points:1.00

Which of the following is not a component of a pressure reducing valve?

Body

Spool

Spring

Adjustment Knob

Clamp*

points:1.00

Describe how sequence valves function.

points:1.00

What type of valve is the above a schematic symbol for?

Describe the function of a check valve.

points:1.00

Describe the function of an integral check valve.

points:1.00

What type of valve is the above a schematic symbol for?

points:1.00

Describe the function of pressure reducing valve applications.

points:1.00